@ DEVICE PIC12F683, FCMEN_OFF, IESO_OFF, INTRC_OSC_NOCLKOUT , WDT_OFF, PWRT_ON, MCLR_OFF, PROTECT_ON, BOD_OFF

OSCCON   = %01110001         ' Internal 8MHz osc.

DEFINE OSC 8

' ======= Common Settings =================================

ADCON0 = 0

ANSEL   = 0

CMCON0 = 7                     ' Comparators off.

TRISIO = 0

GPIO

= 0

' ============= PWM Settings =========

CCP1CON =     %00001100             ' CCP1, PWM mode

PR2         =     250                         '

T2CON     =     %00000101             ' TMR2 on, prescaler 1:4

CCPR1L   = 0

Speed VAR CCPR1L ' Speed variablefor LED brightness.

Speed2 VAR BYTE   ' Speed2  variable for fan speed.

' GPIO.0         ' Empty.

' GPIO.1         ' Empty.

' GPIO.2         ' LED is connected to GPIO.2 (on board PWM module)

Btn   VAR GPIO.3   ' Assuming ; the pin is pulled down to GND by 10K resistor, and button pulls it high to Vdd.

Btn2 VAR GPIO.4   ' Second button. But the pin is pulled high internally. So no external pull up or pull down resistor is required.

' Have a button between this pin and GND (unlike the first button).

Fan   VAR GPIO.5   ' Fan motor output.

INPUT Btn2             ' Make the second button pin an input pin.

OPTION_REG.7 = 0 '

WPU.4 = 1               ' Enable internal pull up resisitor for Second button pin (GP4)

PAUSE 10

Speed2 = 0

' First button, btn, controls the LED via onboard PWM module.

' Second button controls the FAN via software PWM via GPIO.5 output pin and GPIO.4 input pin.

' Having the Fan on GPIO.2 and LED on GPIO.5 would be better I think.

Start:

IF Btn = 1 THEN           ' When this button  is pressed, it pulls the pin high.

IF Speed + 85 < 256 THEN       ' 0, 85, 170, 255, 0

Speed = Speed + 85

ELSE

Speed = 0

ENDIF

WHILE Btn = 1           ' When this button is being pressed, have no interruption on Fan control.

PWM Fan,Speed2,1

WEND

ENDIF

IF Btn2 = 0 THEN           ' When this button  is pressed, it pulls the pin low.

IF Speed2 + 85 < 256 THEN       ' 0, 85, 170, 255, 0

Speed2 = Speed2 + 85

ELSE

Speed2 = 0

ENDIF

WHILE Btn2 = 0                 ' When this button is being pressed, have no interruption on Fan control.

PWM Fan,Speed2,1

WEND

ENDIF

PWM Fan,Speed2,1   ' Always drive the fan output.

GOTO Start

END